Houston Flood Inundation Forecasting System

Components:

- Nowcast precip from IMERG (0.1˚)

- Forecast precip from WRF-downscaled GFS precip (0.1˚)

- Hydrological Modeling – SWAT model (input forcing from IMERG and WRF)

- HEC RAS 2D Simulation (using boundary conditions from SWAT output and IMERG+WRF precip)

The workflow of the system is schematically shown below:

Each component is described below.

1. SWAT Model setup

Domain Setup

SWAT model domain extending over the catchment for downstream station

Calibration of SWAT model

The SWAT model was calibrated at one of the stations (marked blue in the figure above) where USGS gage streamflow is available over 2001-2014.

Validation results for SWAT at single station

R2 = 0.40

The highly flashy response of the basin is leading to some of the missed and overestimated peaks

2. WRF Setup

- The namelist configuration files (with WRF domain) are shared on Github in WRF/ folder

- WRF v 4.1.2 was used for simulation

- The automation script, calling WPS and WRF programs is also shared on Github in /WRF/ folder

3. HEC RAS 2D

- Uses boundary conditions at four locations: 3 stations for upstream inflow, 1 station for downstream outflow and precipitation over the catchment, as shown below.

- Lidar DEM of 1m resolution was used for defining the terrain

HEC RAS boundary condition location (UBC1, UBC2, UBC3, DBC, and precip over the whole catchment). Hatched area shows RAS domain, while outer boundary is the catchment

- Shapefiles for boundary conditions (bc1.shp) and RAS domain (flowarea2d_merged.shp) are shared on Github repository in HEC_RAS/RAS_Model/ folder.

- Script automating the SWAT and RAS models along with post-processing the RAS outputs is shared as HEC_RAS/Automate_SWAT_RAS.py